## This file is a general .xdc for the ZYBO Rev B board

## To use it in a project:

## - uncomment the lines corresponding to used pins

## - rename the used signals according to the project

## change the file extension to .xdc

##Clock signal

#set_property -dict { PACKAGE_PIN L16   IOSTANDARD LVCMOS33 } [get_ports { clk }]; #IO_L11P_T1_SRCC_35 Sch=sysclk

#create_clock -add -name sys_clk_pin -period 8.00 -waveform {0 4} [get_ports { clk }];

##Switches

#set_property -dict { PACKAGE_PIN G15   IOSTANDARD LVCMOS33 } [get_ports { sw0[0] }]; #IO_L19N_T3_VREF_35 Sch=SW0

#set_property -dict { PACKAGE_PIN P15   IOSTANDARD LVCMOS33 } [get_ports { rst }];  #IO_L24P_T3_34 Sch=SW1

#set_property -dict { PACKAGE_PIN W13   IOSTANDARD LVCMOS33 } [get_ports { start }]; #IO_L4N_T0_34 Sch=SW2

#set_property -dict { PACKAGE_PIN T16   IOSTANDARD LVCMOS33 } [get_ports { sw[3] }]; #IO_L9P_T1_DQS_34 Sch=SW3

##Buttons

#set_property -dict { PACKAGE_PIN R18   IOSTANDARD LVCMOS33 } [get_ports { btn[0] }]; #IO_L20N_T3_34 Sch=BTN0

#set_property -dict { PACKAGE_PIN P16   IOSTANDARD LVCMOS33 } [get_ports { btn[1] }]; #IO_L24N_T3_34 Sch=BTN1

#set_property -dict { PACKAGE_PIN V16   IOSTANDARD LVCMOS33 } [get_ports { btn[2] }]; #IO_L18P_T2_34 Sch=BTN2

#set_property -dict { PACKAGE_PIN Y16   IOSTANDARD LVCMOS33 } [get_ports { btn[3] }]; #IO_L7P_T1_34 Sch=BTN3

##LEDs

set_property -dict { PACKAGE_PIN M14   IOSTANDARD LVCMOS33 } [get_ports { led[0] }]; #IO_L23P_T3_35 Sch=LED0

set_property -dict { PACKAGE_PIN M15   IOSTANDARD LVCMOS33 } [get_ports { led[1] }]; #IO_L23N_T3_35 Sch=LED1

set_property -dict { PACKAGE_PIN G14   IOSTANDARD LVCMOS33 } [get_ports { led[2] }]; #IO_0_35=Sch=LED2

set_property -dict { PACKAGE_PIN D18   IOSTANDARD LVCMOS33 } [get_ports { led[3] }]; #IO_L3N_T0_DQS_AD1N_35 Sch=LED3

##I2S Audio Codec

#set_property -dict { PACKAGE_PIN K18   IOSTANDARD LVCMOS33 } [get_ports ac_bclk]; #IO_L12N_T1_MRCC_35 Sch=AC_BCLK

#set_property -dict { PACKAGE_PIN T19   IOSTANDARD LVCMOS33 } [get_ports ac_mclk]; #IO_25_34 Sch=AC_MCLK

#set_property -dict { PACKAGE_PIN P18   IOSTANDARD LVCMOS33 } [get_ports ac_muten]; #IO_L23N_T3_34 Sch=AC_MUTEN

#set_property -dict { PACKAGE_PIN M17   IOSTANDARD LVCMOS33 } [get_ports ac_pbdat]; #IO_L8P_T1_AD10P_35 Sch=AC_PBDAT

#set_property -dict { PACKAGE_PIN L17   IOSTANDARD LVCMOS33 } [get_ports ac_pblrc]; #IO_L11N_T1_SRCC_35 Sch=AC_PBLRC

#set_property -dict { PACKAGE_PIN K17   IOSTANDARD LVCMOS33 } [get_ports ac_recdat]; #IO_L12P_T1_MRCC_35 Sch=AC_RECDAT

#set_property -dict { PACKAGE_PIN M18   IOSTANDARD LVCMOS33 } [get_ports ac_reclrc]; #IO_L8N_T1_AD10N_35 Sch=AC_RECLRC

##Audio Codec/external EEPROM IIC bus

#set_property -dict { PACKAGE_PIN N18   IOSTANDARD LVCMOS33 } [get_ports ac_scl]; #IO_L13P_T2_MRCC_34 Sch=AC_SCL

#set_property -dict { PACKAGE_PIN N17   IOSTANDARD LVCMOS33 } [get_ports ac_sda]; #IO_L23P_T3_34 Sch=AC_SDA

##Additional Ethernet signals

#set_property -dict { PACKAGE_PIN F16   IOSTANDARD LVCMOS33 } [get_ports eth_int_b]; #IO_L6P_T0_35 Sch=ETH_INT_B

#set_property -dict { PACKAGE_PIN E17   IOSTANDARD LVCMOS33 } [get_ports eth_rst_b]; #IO_L3P_T0_DQS_AD1P_35 Sch=ETH_RST_B

##HDMI Signals

#set_property -dict { PACKAGE_PIN H17   IOSTANDARD TMDS_33 } [get_ports hdmi_clk_n]; #IO_L13N_T2_MRCC_35 Sch=HDMI_CLK_N

#set_property -dict { PACKAGE_PIN H16   IOSTANDARD TMDS_33 } [get_ports hdmi_clk_p]; #IO_L13P_T2_MRCC_35 Sch=HDMI_CLK_P

#set_property -dict { PACKAGE_PIN D20   IOSTANDARD TMDS_33 } [get_ports { hdmi_d_n[0] }]; #IO_L4N_T0_35 Sch=HDMI_D0_N

#set_property -dict { PACKAGE_PIN D19   IOSTANDARD TMDS_33 } [get_ports { hdmi_d_p[0] }]; #IO_L4P_T0_35 Sch=HDMI_D0_P

#set_property -dict { PACKAGE_PIN B20   IOSTANDARD TMDS_33 } [get_ports { hdmi_d_n[1] }]; #IO_L1N_T0_AD0N_35 Sch=HDMI_D1_N

#set_property -dict { PACKAGE_PIN C20   IOSTANDARD TMDS_33 } [get_ports { hdmi_d_p[1] }]; #IO_L1P_T0_AD0P_35 Sch=HDMI_D1_P

#set_property -dict { PACKAGE_PIN A20   IOSTANDARD TMDS_33 } [get_ports { hdmi_d_n[2] }]; #IO_L2N_T0_AD8N_35 Sch=HDMI_D2_N

#set_property -dict { PACKAGE_PIN B19   IOSTANDARD TMDS_33 } [get_ports { hdmi_d_p[2] }]; #IO_L2P_T0_AD8P_35 Sch=HDMI_D2_P

#set_property -dict { PACKAGE_PIN E19   IOSTANDARD LVCMOS33 } [get_ports hdmi_cec]; #IO_L5N_T0_AD9N_35 Sch=HDMI_CEC

#set_property -dict { PACKAGE_PIN E18   IOSTANDARD LVCMOS33 } [get_ports hdmi_hpd]; #IO_L5P_T0_AD9P_35 Sch=HDMI_HPD

#set_property -dict { PACKAGE_PIN F17   IOSTANDARD LVCMOS33 } [get_ports hdmi_out_en]; #IO_L6N_T0_VREF_35 Sch=HDMI_OUT_EN

#set_property -dict { PACKAGE_PIN G17   IOSTANDARD LVCMOS33 } [get_ports hdmi_scl]; #IO_L16P_T2_35 Sch=HDMI_SCL

#set_property -dict { PACKAGE_PIN G18   IOSTANDARD LVCMOS33 } [get_ports hdmi_sda]; #IO_L16N_T2_35 Sch=HDMI_SDA

##Pmod Header JA (XADC)

#set_property -dict { PACKAGE_PIN N15   IOSTANDARD LVCMOS33 } [get_ports { ja_p[0] }]; #IO_L21P_T3_DQS_AD14P_35 Sch=JA1_R_p

#set_property -dict { PACKAGE_PIN L14   IOSTANDARD LVCMOS33 } [get_ports { ja_p[1] }]; #IO_L22P_T3_AD7P_35 Sch=JA2_R_P

#set_property -dict { PACKAGE_PIN K16   IOSTANDARD LVCMOS33 } [get_ports { ja_p[2] }]; #IO_L24P_T3_AD15P_35 Sch=JA3_R_P

#set_property -dict { PACKAGE_PIN K14   IOSTANDARD LVCMOS33 } [get_ports { ja_p[3] }]; #IO_L20P_T3_AD6P_35 Sch=JA4_R_P

#set_property -dict { PACKAGE_PIN N16   IOSTANDARD LVCMOS33 } [get_ports { ja_n[0] }]; #IO_L21N_T3_DQS_AD14N_35 Sch=JA1_R_N

#set_property -dict { PACKAGE_PIN L15   IOSTANDARD LVCMOS33 } [get_ports { ja_n[1] }]; #IO_L22N_T3_AD7N_35 Sch=JA2_R_N

#set_property -dict { PACKAGE_PIN J16   IOSTANDARD LVCMOS33 } [get_ports { ja_n[2] }]; #IO_L24N_T3_AD15N_35 Sch=JA3_R_N

#set_property -dict { PACKAGE_PIN J14   IOSTANDARD LVCMOS33 } [get_ports { ja_n[3] }]; #IO_L20N_T3_AD6N_35 Sch=JA4_R_N

##Pmod Header JB

#set_property -dict { PACKAGE_PIN T20   IOSTANDARD LVCMOS33 } [get_ports { jb_p[0] }]; #IO_L15P_T2_DQS_34 Sch=JB1_p

#set_property -dict { PACKAGE_PIN U20   IOSTANDARD LVCMOS33 } [get_ports { jb_n[0] }]; #IO_L15N_T2_DQS_34 Sch=JB1_N

#set_property -dict { PACKAGE_PIN V20   IOSTANDARD LVCMOS33 } [get_ports { jb_p[1] }]; #IO_L16P_T2_34 Sch=JB2_P

#set_property -dict { PACKAGE_PIN W20   IOSTANDARD LVCMOS33 } [get_ports { jb_n[1] }]; #IO_L16N_T2_34 Sch=JB2_N

#set_property -dict { PACKAGE_PIN Y18   IOSTANDARD LVCMOS33 } [get_ports { jb_p[2] }]; #IO_L17P_T2_34 Sch=JB3_P

#set_property -dict { PACKAGE_PIN Y19   IOSTANDARD LVCMOS33 } [get_ports { jb_n[2] }]; #IO_L17N_T2_34 Sch=JB3_N

#set_property -dict { PACKAGE_PIN W18   IOSTANDARD LVCMOS33 } [get_ports { jb_p[3] }]; #IO_L22P_T3_34 Sch=JB4_P

#set_property -dict { PACKAGE_PIN W19   IOSTANDARD LVCMOS33 } [get_ports { jb_n[3] }]; #IO_L22N_T3_34 Sch=JB4_N

##Pmod Header JC

#set_property -dict { PACKAGE_PIN V15   IOSTANDARD LVCMOS33 } [get_ports { jc_p[0] }]; #IO_L10P_T1_34 Sch=JC1_P

#set_property -dict { PACKAGE_PIN W15   IOSTANDARD LVCMOS33 } [get_ports { jc_n[0] }]; #IO_L10N_T1_34 Sch=JC1_N

#set_property -dict { PACKAGE_PIN T11   IOSTANDARD LVCMOS33 } [get_ports { jc_p[1] }]; #IO_L1P_T0_34 Sch=JC2_P

#set_property -dict { PACKAGE_PIN T10   IOSTANDARD LVCMOS33 } [get_ports { jc_n[1] }]; #IO_L1N_T0_34 Sch=JC2_N

#set_property -dict { PACKAGE_PIN W14   IOSTANDARD LVCMOS33 } [get_ports { jc_p[2] }]; #IO_L8P_T1_34 Sch=JC3_P

#set_property -dict { PACKAGE_PIN Y14   IOSTANDARD LVCMOS33 } [get_ports { jc_n[2] }]; #IO_L8N_T1_34 Sch=JC3_N

#set_property -dict { PACKAGE_PIN T12   IOSTANDARD LVCMOS33 } [get_ports { jc_p[3] }]; #IO_L2P_T0_34 Sch=JC4_P

#set_property -dict { PACKAGE_PIN U12   IOSTANDARD LVCMOS33 } [get_ports { jc_n[3] }]; #IO_L2N_T0_34 Sch=JC4_N

##Pmod Header JD

#set_property -dict { PACKAGE_PIN T14   IOSTANDARD LVCMOS33 } [get_ports { jd_p[0] }]; #IO_L5P_T0_34 Sch=JD1_P

#set_property -dict { PACKAGE_PIN T15   IOSTANDARD LVCMOS33 } [get_ports { jd_n[0] }]; #IO_L5N_T0_34 Sch=JD1_N

#set_property -dict { PACKAGE_PIN P14   IOSTANDARD LVCMOS33 } [get_ports { jd_p[1] }]; #IO_L6P_T0_34 Sch=JD2_P

#set_property -dict { PACKAGE_PIN R14   IOSTANDARD LVCMOS33 } [get_ports { jd_n[1] }]; #IO_L6N_T0_VREF_34 Sch=JD2_N

#set_property -dict { PACKAGE_PIN U14   IOSTANDARD LVCMOS33 } [get_ports { jd_p[2] }]; #IO_L11P_T1_SRCC_34 Sch=JD3_P

#set_property -dict { PACKAGE_PIN U15   IOSTANDARD LVCMOS33 } [get_ports { jd_n[2] }]; #IO_L11N_T1_SRCC_34 Sch=JD3_N

#set_property -dict { PACKAGE_PIN V17   IOSTANDARD LVCMOS33 } [get_ports { jd_p[3] }]; #IO_L21P_T3_DQS_34 Sch=JD4_P

#set_property -dict { PACKAGE_PIN V18   IOSTANDARD LVCMOS33 } [get_ports { jd_n[3] }]; #IO_L21N_T3_DQS_34 Sch=JD4_N

##Pmod Header JE

#set_property -dict { PACKAGE_PIN V12   IOSTANDARD LVCMOS33 } [get_ports { je[0] }]; #IO_L4P_T0_34 Sch=JE1

#set_property -dict { PACKAGE_PIN W16   IOSTANDARD LVCMOS33 } [get_ports { je[1] }]; #IO_L18N_T2_34 Sch=JE2

#set_property -dict { PACKAGE_PIN J15   IOSTANDARD LVCMOS33 } [get_ports { je[2] }]; #IO_25_35 Sch=JE3

#set_property -dict { PACKAGE_PIN H15   IOSTANDARD LVCMOS33 } [get_ports { je[3] }]; #IO_L19P_T3_35 Sch=JE4

#set_property -dict { PACKAGE_PIN V13   IOSTANDARD LVCMOS33 } [get_ports { je[4] }]; #IO_L3N_T0_DQS_34 Sch=JE7

#set_property -dict { PACKAGE_PIN U17   IOSTANDARD LVCMOS33 } [get_ports { je[5] }]; #IO_L9N_T1_DQS_34 Sch=JE8

#set_property -dict { PACKAGE_PIN T17   IOSTANDARD LVCMOS33 } [get_ports { je[6] }]; #IO_L20P_T3_34 Sch=JE9

#set_property -dict { PACKAGE_PIN Y17   IOSTANDARD LVCMOS33 } [get_ports { je[7] }]; #IO_L7N_T1_34 Sch=JE10

##USB-OTG overcurrent detect pin

#set_property -dict { PACKAGE_PIN U13   IOSTANDARD LVCMOS33 } [get_ports otg_oc]; #IO_L3P_T0_DQS_PUDC_B_34 Sch=OTG_OC

##VGA Connector

#set_property -dict { PACKAGE_PIN M19   IOSTANDARD LVCMOS33 } [get_ports { vga_r[0] }]; #IO_L7P_T1_AD2P_35 Sch=VGA_R1

#set_property -dict { PACKAGE_PIN L20   IOSTANDARD LVCMOS33 } [get_ports { vga_r[1] }]; #IO_L9N_T1_DQS_AD3N_35 Sch=VGA_R2

#set_property -dict { PACKAGE_PIN J20   IOSTANDARD LVCMOS33 } [get_ports { vga_r[2] }]; #IO_L17P_T2_AD5P_35 Sch=VGA_R3

#set_property -dict { PACKAGE_PIN G20   IOSTANDARD LVCMOS33 } [get_ports { vga_r[3] }]; #IO_L18N_T2_AD13N_35 Sch=VGA_R4

#set_property -dict { PACKAGE_PIN F19   IOSTANDARD LVCMOS33 } [get_ports { vga_r[4] }]; #IO_L15P_T2_DQS_AD12P_35 Sch=VGA_R5

#set_property -dict { PACKAGE_PIN H18   IOSTANDARD LVCMOS33 } [get_ports { vga_g[0] }]; #IO_L14N_T2_AD4N_SRCC_35 Sch=VGA_G0

#set_property -dict { PACKAGE_PIN N20   IOSTANDARD LVCMOS33 } [get_ports { vga_g[1] }]; #IO_L14P_T2_SRCC_34 Sch=VGA_G1

#set_property -dict { PACKAGE_PIN L19   IOSTANDARD LVCMOS33 } [get_ports { vga_g[2] }]; #IO_L9P_T1_DQS_AD3P_35 Sch=VGA_G2

#set_property -dict { PACKAGE_PIN J19   IOSTANDARD LVCMOS33 } [get_ports { vga_g[3] }]; #IO_L10N_T1_AD11N_35 Sch=VGA_G3

#set_property -dict { PACKAGE_PIN H20   IOSTANDARD LVCMOS33 } [get_ports { vga_g[4] }]; #IO_L17N_T2_AD5N_35 Sch=VGA_G4

#set_property -dict { PACKAGE_PIN F20   IOSTANDARD LVCMOS33 } [get_ports { vga_g[5] }]; #IO_L15N_T2_DQS_AD12N_35 Sch=VGA=G5

#set_property -dict { PACKAGE_PIN P20   IOSTANDARD LVCMOS33 } [get_ports { vga_b[0] }]; #IO_L14N_T2_SRCC_34 Sch=VGA_B1

#set_property -dict { PACKAGE_PIN M20   IOSTANDARD LVCMOS33 } [get_ports { vga_b[1] }]; #IO_L7N_T1_AD2N_35 Sch=VGA_B2

#set_property -dict { PACKAGE_PIN K19   IOSTANDARD LVCMOS33 } [get_ports { vga_b[2] }]; #IO_L10P_T1_AD11P_35 Sch=VGA_B3

#set_property -dict { PACKAGE_PIN J18   IOSTANDARD LVCMOS33 } [get_ports { vga_b[3] }]; #IO_L14P_T2_AD4P_SRCC_35 Sch=VGA_B4

#set_property -dict { PACKAGE_PIN G19   IOSTANDARD LVCMOS33 } [get_ports { vga_b[4] }]; #IO_L18P_T2_AD13P_35 Sch=VGA_B5

#set_property -dict { PACKAGE_PIN P19   IOSTANDARD LVCMOS33 } [get_ports vga_hs]; #IO_L13N_T2_MRCC_34 Sch=VGA_HS

#set_property -dict { PACKAGE_PIN R19   IOSTANDARD LVCMOS33 } [get_ports vga_vs]; #IO_0_34 Sch=VGA_VS